Ubuntu 20.04

Account name: uclp

Pass: security

Root Password: toor

VirtualBox tools has been installed

Software Installed:

- Git

- Vim

- curl

- Sublime-text

- pymavlink and mavproxy (sudo pip3 install --upgrade pymavlink mavproxy)

  - This gives the log analysis tools (pyulog,ulog2csv) for mavlink logs from QGroundControl

  - https://github.com/PX4/pyulog

  - https://www.ardusub.com/reference/data-logging.html

- Set up PX4 Autopilot environment for SILT and HILT in folder Documents

- Download the source package from https://github.com/PX4/PX4-Autopilot

  - git clone https://github.com/PX4/PX4-Autopilot.git

- Run the ubuntu.sh with no arguments (in a bash shell) to install everything:

- bash ./Tools/setup/ubuntu.sh

- Install QGroundControl (https://docs.qgroundcontrol.com/master/en/getting_started/download_and_install.html)

- Before installing QGroundControl for the first time:

  - On the command prompt enter:

    - sudo usermod -a -G dialout $USER

    - sudo apt-get remove modemmanager -y

    - sudo apt install gstreamer1.0-plugins-bad gstreamer1.0-libav gstreamer1.0-gl -y

  - Logout and login again to enable the change to user permissions.

  - Download QGroundControl.AppImage for Linux

  - Install (and run) using the terminal commands:

    - chmod +x ./QGroundControl.AppImage

    - ./QGroundControl.AppImage  (or double click)

- Run SILT with gazebo (https://docs.px4.io/master/en/simulation/gazebo.html)

The supported vehicles and make commands are listed below (click links to see vehicle images).

- To run the SILT, start QGroundControl, then start the gazebo-based SILT with the drone model you select.

Load Gazebo Worlds(http://docs.px4.io/master/en/simulation/gazebo.html#loading-a-specific-world) (https://docs.px4.io/master/en/simulation/gazebo_worlds.html)

SILT for mult-vehicle can be enabled by JMAVSim (https://docs.px4.io/master/en/simulation/multi_vehicle_jmavsim.html)

Multi-Vehicle Simulation with Gazebo (https://docs.px4.io/master/en/simulation/multi_vehicle_simulation_gazebo.html)

Simulating GPS Noise (http://docs.px4.io/master/en/simulation/gazebo.html#simulating-gps-noise)

What to develop:

How to simulate different types of attacks:

- Launch GPS Spoofing

  - By modifying the gazebo_gps_plugin.cpp in Tools/sitl_gazebo/src, line 304- 306

    - gps_msg.set_latitude_deg(latlon.first * 180.0 / M_PI);

    - gps_msg.set_longitude_deg(latlon.second * 180.0 / M_PI);

    - gps_msg.set_altitude(pos_W_I.Z() + alt_home_ - noise_gps_pos_.Z() + gps_bias_.Z());

  - Below are images before and after the GPS Spoofing attack

- Launch GPS Jamming

  - For line 36, set a counter for GPS jamming.

  - From line 334 to 356, if the counter reach 0, GPS signal lost for the UAV.

- Control Command Spoofing

  - To create attacks, we need to modify code in /Users/jiawei/Documents/px4/PX4-Autopilot/src/modules/mc_pos_control/MulticopterPositionControl.cpp

  - This is for multicopter position control.

  - Control positions are set in method (line 196) MulticopterPositionControl::set_vehicle_states(const float &vel_sp_z)

- ADS-B Spoofing and Jamming.

- MAVLink Security

Interface Development Idea:

- Develop a command line interface that list different types of attacks

- When a particular type of attacks are selected, the program will modify the code to simulate the attack.

- Provide a second-level of manuals for the same level of attacks.

- Provide the option to restore the documents.